Describe the operation/functionality of:

All Applications -

People Presentation – HTTP/HTTPS, compression, decompression, encryption, decryption . PRESENTING the data.

Seem - Session –  establish, maintain and synchronise interaction between systems.

To - Transport – deals with TCP/UDP, segementation, flow control, windowing, checksum, multiplexing – how much data to send, how and where it goes

Need - Network – IP, host destination, subnet,

Data – Data Link – ARP/MAC, VLAN

Processing – Physical- mediums – ethernet, cabling standards

TCP/IP =

IP – the Identification header in IP identifies which fragments below to the same IP packet.

Public

| Class A | 1.0.0.0-127.255.255.255 |
| --- | --- |
| Class B | 128.0.0.0-191.255.255.255 |
| Class C | 192.0.0.0-223.255.255.255 |
| Multicast | 224.0.0.0-239.255.255.255 |
| Reserved | 240.0.0.0-255.255.255.255 |

Private address spaces are:

10.0.0.0/8

172.16.0.0/12

192.168.0.0/16

Network address i.e. 10.0.0.0/8

Broadcast address (all bits 1) 10.255.255.255/8

VLSM = 172.16.1.0/27 = 5 bits to make 32 = 32 -2 (host and broadcast address)=30 usable host IPs

To calculate the available hosts from a subnet mask…remember to figure out the /Subnet mask i.e. 255.255.255.192= /27 = 5 bits =30 IP addresses (32 -2 ).

IPv6 (unicast, anycast, multicast)

Link  Local = FF80

Multicast: FE

SLAAC = Rules for converting an address to SLAAC

1. Insert FFFE into the middle of the MAC address.

2. Invert the 7th bit, which is usually the second number in the mac i.e. 0300 = 0000 = because 0000=16 bits so 0-4 bits. 7th bit is the binary bit i.e. number 2. Remember, the binary is numbered (left to right) 8420/1. So 1= 0001, 2=0020 5=0101 and A-F is 9-14.

3. Zero out the leading zero’s

4. Reduce 1 block of 0’s in the address to ::  (can only be done once)

TCP:  reliability - error checking, multiplexing – socket creation for multiple connections to the same host (sockets determine which application a connection belongs to), connection establishment -3 way handshake, flow control – sequence bits/windowing.

UDP: connectionless, no delivery method – DNS: 53, DHCP:67, NTP:123, SNMP:161 all UDP.

ICMP: Can be used in UDP port scanning attacks – an attacker will run a scan by sending ICMP against port ranges and try to see a ‘Host unreachable’ response to show the port is open.

ARP

DNS

Hierarchal architecture called domain namespace. Leafs = a specific resource and identified by their FQDN.

DNS Resolver = A client sending a recursive message to the servers. The server(s) send iterative messages to the other servers (remember, only clients send recursives).

It goes:

ROOT .

Top level domain: 		 .com , .net , .co.uk, .us,

Second level domains:  		cisco, iana, amazon

Subdomain:			 amazon.sales.net, cisco.telephony.com

Resource name: 			www.

DHCP

Router

Layer  3 device, ACL

Switch – full duplex, and each port identifies a collision domain. Layer  2 or 3. MAC flooding, VLAN’s can isolate broadcast  domains to be per-interface.

STP – lowest bid wins, root ports, DP, non-DP,

MAC Address – 48 bits. 24 bits OUI assigned (left, company) vs 24bit Vendor (right, unique device).

Hub – creates a collision domain by regenerating a signal (think repeater). One hub and it’s connected devices create 1 collision domain. CSMA/CD uses backoff algorithm to prevent sending at same time.

Bridge – help separate collision domains.  A bridge between 2  hubs creates 2 separate collision domains.

RIPv2,OSPF and EIGRP are CLASSLESS protocols. IGRP is classful.

OSPF has information of the whole topology..

Wireless access point (WAP)

Autonomous AP – single access point

LWAP – lightweight access point uses WAPCAP to create a tunnel back to the WLC. The WLC deals with QoS, authentication, authorisation and encryption and session level data.

War driving – driving around seeking SIDs

Evil Twin – duplicate WAP

Wireless LAN controller (WLC)

WLC can handle the AAA process. Typically the WLC sits near the distribution layer?

Describe the functions of these network security systems as deployed on the host, network, or the cloud:

1.5.a Firewall

Have 3 flavours:

Traditional – packet filtering with ACLs, NAT.

Stateful inspection – Track every packet as it passes through and ensures the connections being made in/out of the network are valid. It also tracks application layer information.  They can be configured  for active-active, active-standby or clustered firewalling,

Next-Gen Firewalls – Cisco ASA – has the ability to:

- NAT

- L2 ‘transparent’ or L3

- Stateful inspection + packet filtering

- Context aware filtering – i.e. it can track the conversations between applications in and outside the network to ensure the flows are valid.

Firewalls segregate networks by creating DMZ’s – webserver farms/extranet connections are common uses of DMZ.

Host Based

Cisco ASA can be ran in a routed mode (i.e. a layer 3 firewall where it is a hop in a route) or transparent mode (layer 2 – not seen as a routed hop in a network path, also known as a stealth firewall and has the same network on its outside and inside interface).

Host based firewalls ‘personal firewalls’ control layer 3 and 4 based information (Network and transport).

1.5.b Cisco Intrusion Prevention System (IPS)

IDS = Intruder Detection System is not prevention – they alert to suspicious activity based on signatures or anomalies in the traffic.

Typically, an IDS analyses a copy of a packet to inspect it and can work with firewall/other network devices to detect an attack i.e. reactive.

IPS = Intruder Prevention System – can drop malicious packets if needed (inline mode), or operate in a promiscuous mode = monitoring to assess impact on a network. IPS tends to offer full packet inspection and can be a drain on resources. But can PREVENT attacks i.e. pro-active.

False positive: legit traffic flagged as negative.

False negative: fails to recognise a malicious event. V dangerous.

True positive: correct behaviour when an actual threat is detected.

True negative: when no event is triggered for legit traffic.

IPS come in 3 flavours:

Traditional Network-based IPS:

i.e. these deal with the following methods for analysing traffic:

- Pattern Matching – can cause false positives, tracks things like traffic packet size

- Protocol analysis – inspects header information for protocols looking for ‘what to expect’ i.e. SMTP header info like MAILTO etc.

- Heuristic analysis – needs fine tuning per environment, CPU heavy, spots patterns based on trends in traffic.

- Anomaly based – traffic that deviates from the norm

Tradition based firewalls aren’t good for spotting DDoD attacks that come through the network as valid packets (think reflected attacks for example, or SYNACK attacks) . Also not good for  zero-day attacks.

Next-Gen IPS- NGIPS

- Global correlation capabilities – NGIPS generally have all of the above, plus Cloud capabilities making use of Talos to enable the NGIPS to filter traffic based on reputation .

Firepower Management Console FMC

Cisco Firewpower 7000/8000 appliance and NGIPSv for VMWare.

Cisco Firepower Threat Defence – is a type of IPS. Remember:

1.5.c Cisco Advanced Malware Protection (AMP)

ClamAV (opensource cisco Malware)

Immunet (free) – (non open source) F-Secure, Panda, Avast are other weird anti malware offerings.

Cisco AMP for networks can be installed alongside FirePOWER applications or firewalls (ASA) or as a standalone appliance, It query ThreatGrid (cloud based) with a hash (SHA-256) of a file that could be malicious. Has a glovebox feature that sandboxes malware but also interacts with it (which is often not possible)

1.5.d Web Security Appliance (WSA) / Cisco Cloud Web Security (CWS)

Cisco’s proxy offering – can be ran in explicit proxy mode or transparent (clients not aware their requests go via proxy).

1.5.e Email Security Appliance (ESA) / Cisco Cloud Email Security (CES)

Cloud-based e-mail security run on AsyncOS – this can effectively provide outsource e-mail security in the cloud. Preventing against:

Phising, Spear Phishing, Whaling (CEO/Board level people targeted)

1.6 Describe IP subnets and communication within an IP subnet and between IP subnets

Layer 3 device required (SVI, ROAS) to route between VLANs. Trunk links transport traffic between VLANs.

1.7 Describe the relationship between VLANs and data visibility

VLANs can use ISL or 801.Q (dot1q) for encapsulation. They help segment networks and prevent pivot attacks, broadcast storms.

1.8 Describe the operation of ACLs applied as packet filters on the interfaces of network devices

Router ACL’s outbound/inbound, drop-all, extended or standard. Extended can filter based on L2-L4. Ports can have security levels assigned to them from 0-100 (most secure) – ports with higher levels can pass traffic to lower, but you can also filter based on security level. Packets are evaluated from top-down so ordering is vital, the first rule that matches will determine it’s outcome.

1.9 Compare and contrast deep packet inspection with packet filtering and stateful firewall operation

Deep Packet Inspection (DPI) – inspects payloads at Layer 7 (presentation) and tailor a rule to, say, deny P2P applications using HTTP protocol (layer 7). Cisco ASA

usually a mirror port takes a copy of the data . No other level of detail is possible. Wireshark,

Stateful firewalls keep a state-table and trace packets to ensure the connections they are flowing along are maintained and

are context-aware and can do in-line inspection of packets. Cisco ASA can be deployed as a L2 firewall (ports have IP addresses), or a Stateful firewall if used with Firepower Defence

1.10 Compare and contrast inline traffic interrogation and taps or traffic mirroring

Inline = real time inspection of packets – high system resource overhead

Mirroring = SPAN style – takes a copy of traffic and

1.11 Compare and contrast the characteristics of data obtained from taps or traffic mirroring and NetFlow in the analysis of network traffic

- NetFlow is able to capture traffic flows based on metadata information for traffic analysis, costing, billing etc .

- It can help detect misconfigured firewalls

- Monitor application usage

- Source/Destination IP and Port type,Tos field, L3 protocol type.

- IPFIX is a reporting standard that defines how data is collected from network devices and provides a template to feed into other systems. A collector (i.e receiver of netflow data) can receive netflow data with no initiation required for the connect to take place. Uses

- NetFlow Reliable Export With SCTP feature in Cisco IOS Release 12.4 replaced using UDP – SCTP buffers messages on a router until they can be sent and has congestion control built in,

- IPFIX was based on version 9 of Netflow export format

1.12 Identify potential data loss from provided traffic profiles

17%

2.0 Security Concepts

2.1 Describe the principles of the defence in depth strategy

Taking a holistic approach to security – not just relying on one expensive  system. Thinking about all aspects –

Non technical – policies and procedures

physical (barriers/walls, gates)

network (IPS/IDS, ACLs, packet capture, Netflow, Syslog)

host (AV, firewall, AMP)

Application security – testing of apps, defence against XSS, SQL injection etc

The data traversing the network – encrypted, encryption at rest

Think about the 5 planes: Management, Control, Data, Services (layer 7 app flow) and Policies (business requirements, policies and procedures_.

Identitification: unique, nondescriptive (i.e. 5eet35 is more secure than Jane’s ID badge) and securely issued.

Military: Top Secret, Secret, Confidential, Unclassified

Commercial: Confidential, Private, Sensitive, Public

Damage degree: Grave, Severe, Damage, Not Significant (left to right)

Types of control:

Preventative – i.e. a fence, passwords and access lists.

Deterrent – intended to deter an incident from occurring i.e. warning banner logging into a switch

Detective Controls – i.e. a SIEM system, detecting an attacker

Corrective Controls – Quarantining an infected computer, sandboxing a malware

Recovery controls – Performing a backup, creating a DR plan

Compensating controls – security guard checking your badge because  the reader is out of order – intended to  substitute if a primary control is not in place.

2.2 Compare and contrast these concepts

2.2.a Risk

A potential threat.

2.2.b Threat

A threat to any asset. Threat actors = script kiddies, government sponsored , hacktivists,  terrorist g roups, organised crime.

2.2.c Vulnerability

Vulnerabilities are a known or unknown weakness  in a system or design. MITRE maintains the CVE(common vulnerabilities  and exposures)  library of vulnerabilities and vendors can apply for  a CV number to register a known vulnerability.

Reconnaisance  attacks (port scanning, SYNACK attacks, traffic fragmentation, DoS, etc) can all look for vulnerabilities.

Vulnerability scanners include Nessus, Nexpose, AppScan, nmap

White box = Internal attack – ACTUALLY THE BEST OPTION! Hacker knows about the network, so the scenario might/would fail in  a black box/zero knowledge attack.

Black Box  = aka no knowledge external threat scenario.

Gray box = has some reconnaissance level intel i.e. public information, DNS info etc.

2.2.d Exploit

Software or a sequence of commands that takes advantage of a vulnerability to cause harm to a system or network. Anger, Mpack, Blackhole, Fiesta are examples of exploits.

2.3 Describe these terms

2.3.a Threat actor – script kiddies, state sponsored, hactivists, terrorists. People who have malicious intent.

2.3.b Run book automation (RBA)

Run book automation is the process of automating operational, security, infrastructure tasks i.e responding to a security event or automating a monitoring process. The metrics for measuring RBA are:

- MTBF mean time between failure

- MTTR mean time to repair

- Mean time to control/mitigate security event.

2.3.c Chain of custody (evidentiary)

Involves the processes used to preserve evidence after a security event – how is it collected, how is it transported/stored, how is it recorded, what  is collected.

2.3.d Reverse engineering

Includes decompilers (decompile an .exe to make machine readable code),  debuggers, disassemblers.

2.3.e Sliding window anomaly detection

A profiling time window PTF is a period where your traffic is monitored to create a ‘normal use’ profile. This is a sliding window, default is usually 1 week. If traffic deviates from this profile, it could be considered an anomaly.

2.3.f PII

Personal identifiable Information – name, address, drivers licence, passport number etc. Can also include biological information (xray , characteristics, photos, fingerprints).

2.3.g PHI – HIPA require organisation to adopt security regs for protecting this info.

Protected Health Information – DNA, blood type, medical history, age, gender, drivers licence number.

2.4 Describe these security terms

2.4.a Principle of least privilege – giving the minimal amount of permission to perform the required task. This reduces privilege creep and is required for users as well as system processes (e.g. you don’t might restrict an application from running with root level permissions).

2.4.b Risk scoring/risk weighting

A general framework towards risk and how companies should approach cyber risks can be adopted in 2 parts:

Inherent Risk Profile and Cybersecurity Maturity – this identifies the risks before implementing any changes and scores risks across 5 maturity levels.

ISO 27001  organisation of standards- 27001 focusses on risk based planning- identifying if the responses are sufficient given the nature  of the potential threat.

Scoring/weighting:

Can be done using the CVSS scoring system.

2.4.c Risk reduction

2.4.d Risk assessment

Port scanning (active passive), acceptance of risks

2.5 Compare and contrast these access control models

2.5.a Discretionary access control

DAC - The owner of the file provides you (or an AD group) access on a need-to-know basis using the principle of least privilege. Privilege creep can occur.

2.5.b Mandatory access control –

MAC – Based on assets/objects being marked and with a classification and category i.e.  Top Secret / Sales Department. The OS enforces the rights and a policy enforcer decides who to grant access to.  Provides strong data security.

2.5.c Nondiscretionary access control

NDAC – RBAC – role or function of a subject determines their access  rights, permissions are assigned to a job role and are sufficient to perform a task.

ABAC – attribute based access control i.e. 3rd floor staff in sales with Confidential rights. Ties together many attributes to grant access. Environmental attributes would be user location.

Access controls can also be: non-technical (policies, procedures and training), physical, technical (or Logical) network ACL’s, AAA.

A FAR is false acceptance rate = Letting in the wrong people.

FRR = Rejecting valid people.

False positives = rejecting positive/legit traffic

False negative = failing to recognise malicious traffic

True positive = correct behaviour – recognising malicious traffic.

True negative = normal behaviour when no event has occurred.

2.6 Compare and contrast these terms

2.6.a Network and host antivirus

Network antivirus can stop attacks at the point of entry. Host antivirus (Immunet (cisco), AVG are free).

3 2.6.b Agentless and agent-based protections

Agent based will require signature updates, agentless

2.6.c SIEM and log collection

Security Information and Event Monitor

An event is any observable occurrence  in a system.

A security incident is any event that breaches security policy.

Characteristics of a SIEM

- Log Collection – collects logs from multiple sources

- Normalisation – pick the attributes from log files into a standard template/model.

- Aggregation – remove duplicates based on common information.

- Correlation – this is what defines how effective a SIEM is, how a system associates events and creates an actionable event for a sec admin to investigate.

- Reporting – real time monitoring, historical reports.

Security Incident Monitor – deals

Splunk, Graylog and ELK Stack, HP Arcsight, Logrhythm, NetIQ, QRadar  are open source log collection and analysis platforms. Normalisation is the process of removing duplicate, ‘known’ data or non-events . Correlation is the feature that defines next-gen  SIEM systems and pulls together data from numerous sources to create a security view of the network.

2.7 Describe these concepts

2.7.a Asset management

Marking assets based on their classification and category. But also asset management entails having an asset usage policy (i.e. laptop use) and who is responsible for the assets – typically the Asset Owner is responsible for destruction and classification of an asset.

2.7.b Configuration management

Configuration management is a CMDB – this changes as and when RFC’s are completed , when systems are  changed this should be recorded int the CMDB.

2.7.c Mobile device management

Includes device policies, provisioning and enrolling the devices and ensuring baselines are met (OS, patch level etc) comply with network policy. Cisco ISE can provide these policy configurations along with an MDM server like Airwatch.

2.7.d Patch management

Agent-based patch management will automatically apply a patch (think SCCM). Patches should be raised as an RFC, tested and verified before being deployed. After they are deployed, the CMDB will be updated as the RFC is closed out.

Standard change is a maintenance or repeatable task that already authorised.

Emergency change

Normal change undergoes the normal level of scrutiny.

2.7.e Vulnerability management

The CVE common vulnerabilities and exposures register keeps data on system, device, OS vulnerabilities. People can apply for a  CV number to submit new ones. Vulnerability scanners  can be active or passive and work with common/known vulnerabilities to assess the weaknesses in a system. A passive scan is deployed over the network and looks for trends to spot infections.

12% 3.0 Cryptography

3.1 Describe the uses of a hash algorithm

You can’t produce a different hash from the same block of data aka COLLISION RESISTANCE. A rainbow table is a series of passwords and hashes that are stored and used to try and crack a hashing algorithm. Hash algorithms are used in Digital certificates and ensure data integrity. They must provide variable length-input with fixed length output, and one-way, non-reversible operation.

Weakest ----------------------Strongest

MD5 (128)  		SHA-1(160bit)		SHA2 (224-512)

3.2 Describe the uses of encryption algorithms

Encryption ensures privacy. Encrypted data is difficult to inspect and is challenging for  IPS/IDS systems, it can also be used by attackers to exfiltrate data from a compromised endpoint. Encryption works with authentication and integrity with VPNs.

3.3 Compare and contrast symmetric and asymmetric encryption algorithms

Symmetric = same key to unlock, DES, 3DES, AES. Typically used for data at rest

Asymmetric = VPN peer authentication - system resource heavy, different private keys to unlock = RSA (PKCS#1), Diffie Hellmen (PKCS#3), ECC. Typically used for data in flow.

2 users are issued a digital cert from a mutually trusted CA, the cert has the CA’s public  key, a digital signature and their respective private keys in them. The users also transfer the public keys to each other.

3.4 Describe the processes of digital signature creation and verification

Digital signatures = identity certificates i.e. a cert to identify a device. A digital signature may contain the IP address of the device, FQDN and public key of the issuer.

Root Certificate – identifies a root CA.

First a device has to ‘trust’ it’s CA. In order to do this is needs the public key of the CA but how can it trust the public key? Browsers contain the public key for root CA’s, so for a device to trust the CA, it must request it’s digital signature and then verify it using the native public key. Once the CA cert  is issued, a user can use an out of band method (phonecall, for example) to verify the CA is legit. At which point, it is now trusted.

Digital Signatures = CA

Trusted/root CA’s issue digital signatures that contain public keys.

Verification – hash verification

The sender encrypts a hash of the data with their private key and sends it to the recipient. The receiver then uses the senders public key (which was included in the digital signature) to decrypt the hash and then verifies the rest of the packet – thus confirming integrity of the packet.

3.5 Describe the operation of a PKI

PKI’s are for managing the creation public and private keys, issuing keys/certs, revoking certificates/keys and verifying the identity (nonrepudiation) of a server or device that receives data. The Root CA doesn’t issue certs, the RA registration authority does – this is a subordinate CA.

Public key infrastructure may have a CRL list (cert revocation list) .

3.6 Describe the security impact of these commonly used hash algorithms

3.6.a MD5 – weak

3.6.b SHA-1

3.6.c SHA-256

3.6.d SHA-512 - strongest

3.7 Describe the security impact of these commonly used encryption algorithms and secure communications protocols

3.7.a DES

3.7.b 3DES

3.7.c AES

3.7.d AES256-CTR

3.7.e RSA

3.7.f DSA

3.7.g SSH

SSH uses public key cryptography to authenticate remote computers .

3.7.h SSL/TLS

SSL is widely available in browsers and can redirect requests to HTTP to use SSL.

TLS is transport layer security (and is another term to encompass SSL). SSL can provide a 443 secure tunnel for network access without the need for a client. Clientless or Client-based SSL VPN tunnels exist. With an SSL VP tunnel you would want reverse proxy, port forwarding options setup.

3.8 Describe how the success or failure of a cryptographic exchange impacts security investigation

RADIUS 	vs	 TACACs+  	vs	Diameter

Radius is UDP, password-encrypted (not whole packet) and doesn’t allow for granular command authorization i.e. it’s slightly poo but Cisco generally prefer it for exam. TACACs is Cisco proprietary uses TCP, can encrypt the entire payload and allows granular auth commands.

Diameter is the daddy for CAPABILITIES EXCHANGE and uses -capability-exchange-request CER packets and packets with names like DEVICE WATCHDOG REQUEST. It’s another protocol but offers failover, TLS security, and application IDs.

Diameter uses TCP or STCP protocol

802.1X is used (with RADIUS or Diameter serving as the AAA protocol) to auth users in the Supplicant > Authenticator  > Auth Server model. EAPoL encaps the packet throughout the authentication handshake sequence, with EAP  as the auth protocol.

3.9 Describe these items in regards to SSL/TLS

3.9.a Cipher-suite 3.9.b X.509 certificates

Suite-B is a  collection of the most secure encryption protocols. X.509 certifificates detail how things like LDAP databases are used to authenticate users and their  access on the network.

3.9.c Key exchange

3.9.d Protocol version

3.9.e PKCS

| Public Key Cryptography Standards i.e how CA's issue certs between themselves |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| PKCS #1 | RSA |  |  |  |  |  |
| PKCS#3 | DiffieHellman Key Xchange |  |  |  |  |  |
| PKCS#7 | Response to#10, usually a digital cert, a method for disseminating (sending) certificates. |  |  |  |  |  |
| PKCS#10 | Cert request inc. a public key |  |  |  |  |  |
| PKCS #12 | Public and private key store, uses a password based key to unlock |  |  |  |  |  |

19% 4.0 Host-Based Analysis

4.1 Define these terms as they pertain to Microsoft Windows

4.1.a Processes

A process is a program the system is running made of  one or more threads. A job is a group of processes. Procesess must have permissions within windows.

4.1.b Threads

Thread is a basic unit an OS allocates process time to.

4.1.c Memory allocation

Compile time: Static memory allocations

Run Time: Dynamic/Volatile – i.e. RAM - heaps of memory

Stacks of memory  reserved for thread execution

Virtual address space = virtual memory used by processes

4 4.1.d Windows Registry

Hives: HKCU, HKLM, HKU, HKCR

4.1.e WMI – Millenium onwards. Windows Management Instrumentation . Cannot uninstall applications but can stop/start services, query system information and list processes.

4.1.f Handles – leak can occur if a handle is not released. An abstract reference value to a resource, it also hides the physical memory address from the user.

4.1.g Services – services.msc, sc.exe, permissions needed for services because they run in their own session e.g. you can run a service as another user account, so it has it’s own session.

4.2 Define these terms as they pertain to Linux

4.2.a Processes

Orphan = parent process dies but the child process continues to run

Zombie = hen the memory is released but the processes remains in the entry table.

4.2.b Forks

Fork is when a new process is created (a copy of an existing process)

4.2.c Permissions

These are applied in the order of owner, group, everyone else and in format of rwx, rw, x. They can also be numerical….flesh this out…

4.2.d Symlinks

4.2.e Daemon

A Linux process is a daemon. These are typically started/created by the init process which has an ID of 1, but not always.

Linux stores logs in /var/log

4.3 Describe the functionality of these endpoint technologies in regard to security monitoring

4.3.a Host-based intrusion detection

4.3.b Antimalware and antivirus

AntiMalware/AV defends against: virus’s (malicious software), worms (self-replicating and spreads across network), trojan (can be back doors, usually malicious code injected into legitimate  apps)

4.3.c Host-based firewall

Aka Personal Firewall.

4.3.d Application-level whitelisting/blacklisting

Whitelist / Blacklist / Graylist (not determined but can be moved to white/black with more info).

Can be cumbersome to administer (what’s still in the blacklist? Does it need to be? Etc).

4.3.e Systems-based sandboxing (such as Chrome, Java, Adobe reader)

3 services that provide this:

Chromium

Java jvm

HTML5 ‘sandbox’ with iFrame

They keep the threat to a limited memory space and prevent it from reaching other parts of a system. Cisco AMP receives feeds from Cisco Threat Intelligence to keep signatures/malware protection up to date.

4.4 Interpret these operating system log data to identify an event

4.4.a Windows security event logs

4.4.b Unix-based syslog

/Var/log

Remember that UNIX logs provide you the logging level requested + higher. For example, err level logging would include error, critical, alert, emergency logs.

4.4.c Apache access logs

ErrorLog is where apache writes its errors.

4.4.d IIS access logs

19% 5.0 Security Monitoring

5.1 Identify the types of data provided by these technologies

5.1.a TCP Dump

Open source MAC packet sniffer – full packet analysis. Wireshark is another option.

5.1.b NetFlow

Meta data of IP traffic flows. Can be used to detect anomalies in network and put into IPFIX format to allow ingestion in other systems. IPFIX allows an initiation-less collection of Netflow data from devices to the collector, using SCTP protocol.

Helps to :

- Identify DoS attacks,

- Identify firewall misconfigurations

- View traffic flows and network usage (accounting)

- Quickly view compromised endpoints

Netflow open source analysis tools: Silk, Elk, NFDump

Cisco Netflow analysis tools: Cisco Lancope Stealthwatch – provides NAT-stitching analysis.

5.1.c Next-Gen firewall

Next-gen Firewalls can provide stateful inspection and layer 2-5 inspection as well as access control policies

5.1.d Traditional stateful firewall

Use ACL’s and ACE to filter traffic based on layer 2,3,4 information (IP address source/destination, port (UDP/TCP) protocol and Ethertype and WebType headers.The 5 tuple data)

5.1.e Application visibility and control

5.1.f Web content filtering

Cisco Web Security appliance can act as a proxy – probably event based data (user requests for X website at X time from X IP address) and

5.1.g Email content filtering

Cisco email security uses ThreatGrid data to provide sandboxing and e-mail analysis. E-mails can be encrypted using Pretty Good Privacy PGP encryption, which works for emails and files.

DKIM is domain-key verification for mail i.e. verifying DOMAIN and is better than SMIME. S/MIME is e-mail verification using public key cryptography.

5.2 Describe these types of data used in security monitoring

Netflow data (metadata), host-based logs (application logs, administrator login logs, AMP logs, Web security logs, E-mail security logs, personal firewall logs), external feeds which provide IP address and IoC indicators of Compromise e.g.  CRITS is an open source threat intelligence feed.

Host based logs can be problematic because they’re only as reliable as the last IP address of the host (remember how often DHCP changes addresses on a host? Think IP address turnover….).

5.2.a Full packet capture

Heavy CPU overhead, also requires specialist knowledge to interpret the data but no other data provides this level of details. Wireshark (filters/gui) and TCPDump are examples of these tools.

5.2.b Session data

When the session was created/terminated, what was accessed, who accessed it, how long was it accessed for. Connection methods / RADIUS data.

5.2.c Transaction data

Transaction database files to show changes to a SQL bd

5.2.d Statistical data

5.2.f Extracted content

5.2.g Alert data

Syslog can log events on network infrastructure devices – password attempts, configuration errors.

Tools like SPLUNK and Graylog are useful at scale because they aggregate  syslog messages and can monitor security events to give a network wide view.

NetFlow provides metadata of a the types of traffic  flows in the network and can be combined with Stealthwatch Lancope to monitor for events and alerts. It can also be used with IPFIX to report.

- Monitoring traffic usage/billing/accounting

- Misconfigured firewalls (by showing how traffic is moving)

- Prevent Dos attacks (although typically throttling requests/traffic can do this)

5.3 Describe these concepts as they relate to security monitoring

5.3.a Access control list

5.3.b NAT/PAT

PAT/NAT can mask the source IP – you need Cisco Stealthwatch with NetFlow data  to map the translated IP addresses (NAT stitching).

5.3.c Tunneling

DNS tunnelling occurs when data is being exfiltrated from a compromised system in DNS payloads. DNScapy and DNS..-t and – p these two also provide ability to  detect DNS tunnelling.

5.3.d TOR

Tor exit node – is the gateway back to the internet/exit from the Tor network. Tor uses onion-routing to only reveal a single layer of the OSI model (i.e. source IP) instead of the full IP headers. Packets are routed all over the world to make connections untraceable.

5.3.e Encryption

Makes it difficult to monitor traffic -

5.3.f P2P

Lionshare, Napster, Peercoin etc- P2P can include malware in downloaded or legitimate content.

5.3.g Encapsulation

5.3.h Load balancing

5.4 Describe these NextGen IPS event types aka Cisco Firepower NGIPS

5.4.a Connection event

A user connecting to a networking e.g. the RADIUS auth process, supplicant > authenticator > auth server. Session creation/VPN authentication

5.4.b Intrusion event

5.4.c Host or endpoint event

This could be something like Malware or a virus being detected, or a worm

5.4.d Network discovery event

5.4.e NetFlow event

5.5 Describe the function of these protocols in the context of security monitoring

5.5.a DNS

Attackers can use DNS packets to exfiltrate data by using DNS tunnelling using BASE64 encoding to add sensitive info into DNS packets from a  compromised host and send it out to a name server. Examples of this are DNScapy, dns2tcp.

5.5.b NTP

Show ntp status – shows a paragraph of info about the connection

Show ntp associations – shows peers/servers and the connection addresses + connection status

Security logs and security event info is useless if the time/date is not SYNCHRONISED. NTP will synchronise the clients and servers to prevent this. Difficult to correlate a timeline of a security event if device logs are not in sync.

5.5.c SMTP/POP/IMAP

5.5.d HTTP/HTTPS

21% 6.0 Attack Methods

6.1 Compare and contrast an attack surface and vulnerability

Attack surface is the number of points or vulnerabilities in a  given system that are  accessible to an attacker.Attack vector is the route taken to perform an attack. And a vulnerability is a flaw that can leave a system open to an attack.

6.2 Describe these network attacks

6.2.a Denial of service

Denial of service would perhaps slow traffic down (congest it, after flooding requests) so this would appear as an abnormal traffic flow in NetFlow/Stealthwatch logs. Could also be used against an IPS/IDS to bring it down or create  a window of opportunity.

Direct – Attacking a host by flooding requests , .e.g SYN packet requests to a cloud server- this causes hosting costs to skyrocket.

Reflected – the attacker spoofs the IP of a victim and sends requests to a server, the responses are then sent back to the victim (and not the attacker).

Amplified – the response packet can be much larger than a request – so combined with a reflected attack a victim can be flooded with large, response packets.

6.2.b Distributed denial of service

Botnets, C2 device – orchestrate a botnet (collection of compromised machines) to send spam e-mail/flood traffic to devices etc.

6.2.c Man-in-the-middle

Why? Access control – inspect traffic that is on the network

Layer 2 – ARP-spoofing & STP– Attacker creates a spoof MAC address to act as the default gateway, all traffic routes to the attackers device and (optionally) can be sent on to it’s destination (thus, being stealthy and less likely to raise alarm). Connecting a rogue switch into network to become the root can also cause this. Enabling dynamic arp inspection on the switch can mitigate this.

Layer 3 – Router tables – attacker can access  a router and advertise a ‘better’ route to their device to collect traffic. BGP hi-jacking

Session Hi-jack attack – an attacker can inject code into a users session with a webserver and take control of it.

Malware -  compromise a users machine, install malware – this can cause packets to be sent to an attacker before they are encrypted with SSL/TLS etc.

Trojan Horse – a type of malware that executes instructions to delete files, steal data, compromised the OS. Typically used with a form of social engineering to fool victims into installing software. Trojans can also act as back doors.

Back doors – gaining future access to a system by opening ports and allowing attackers to  gain access remotely.

Rootkit= attack to gain root-level access.

6.3 Describe these web application attacks

6.3.a SQL injection –

injecting SQL queries into web forms in the attempt to query a db.

6.3.b Command injections

Dictionary attacks (password), jailbreaking, rainbow tables (hashing attacks where password combinations and their hashes are forced)

6.3.c Cross-site scripting  - when a trusted  websites hosts malicious content for example entering data into a phony form

6.4 Describe these attacks

6.4.a Social engineering

Leveraging the weakest link – humans. i.e. spoof job offer, attacker requests job details /interviews and asks technical questions, then attacks. Other examples are  phishing, pharming (directing a URL from a valid page  to malicious one) Malvertising – malicious adverts embedded in a web page. To mitigate, password management, 2FA, AMP, change management processes, physical security etc.

6.4.b Phishing

A targeted attack against an individual, usually from e-mail.

Spear Phishing – targets one person, usually preceded by some social engineering

Whaling – CEO/Board Level focussed phishing attempts.

6.4.c Evasion methods

6.5 Describe these endpoint-based attacks

6.5.a Buffer overflows – memory buffer overflows by allocating too much data

6.5.b Command and control (C2) – a C2 system helps orchestrate a Botnet (a collection of breached machines) to perform a DoS attack.

6.5.c Malware

Cisco AMP (advanced malware protection) can provide endpoint protection –

6.5.d Rootkit

6.5.e Port scanning – aka Reconnaissance attacks

Basic  port scan– will actively test UDP and TCP to see which ports are open

TCP scan – tries to initiate TCP handshake on ports

TCP SYNCACK – ‘half opening scan’ – checks to see if the target responds with a SYN-ACK packet.

UDP Scan- uses ICMP because UDP is connectionless to see if the ICMP replies with ‘Host unreachable’

6.5.f Host profiling – see my 210-255 notes on host profiling.

6.6 Describe these evasion methods

6.6.a Encryption and tunneling – Encryption can be a plus for privacy, but a problem if threat actors use it. Tunnelling can be

6.6.b Resource exhaustion – DoS or DDoS can cause resource exhaustion (i.e. forcing a server to respond to SYN ACK packet requests) and this can allow threats to stealth past an IPS. Deterrents to this are throttling to restrict user allocation of bandwidth.

6.6.c Traffic fragmentation –

Off-set values in IP header and Overlapping - Mis-matching the sequence numbers in IP packets to trick a router into expecting more/less packets.

Breaking malicious payloads into smaller packets - Fragmentation– IPS systems have to be able to reevaluate the entire sequence, not just individual packets, so attackers can break packets into fragments.

PROXY SERVERS HELP PREVENT THESE TYPES OF ATTACKS!

6.6.d Protocol-level misinterpretation / Protocol Manipulation – Sending a packet with a TTL of 0, knowing the router would drop that packet and then immediately re-sending with TTL of 100 – the router would expect a re-send of the packet and will allow a potentially malicious packet through.

6.6.e Traffic substitution and insertion -

Substituting data for malicious code that is in different format, but the same meaning e.g ASCII for UNICODE, carriage returns for tab . Security products need to be able to analyse extended characters and unicode deobfuscation to prevent this.

6.6.f Pivot aka island hopping.

Involves gaining access to a  guest network and reaching a device, then creating new access to the network once you’re in. Network segmentation limits quantity of available devices per-network which is a good mitigation. Remember, PIVOT = moving to new, so something like a rootkit (which just gets a foot into the network) is not a  pivot attack. VLAN hopping is.

Prevention?

Cisco ISE – create policy based access  control to say, Julie on iPhone, between 9-5pm from 3rd floor can access XYZ

Patch management – fix vulnerability

Segmentation – VLAN and DMZ

Lancope Stealthwatch

Netflow with Cisco Stealthwatch – this can monitor for unusual activity and provide a starting point for identifying unusual usage patterns.

6.7 Define privilege escalation –

Gaining access or ‘user  level’ access and then jail breaking/dictionary attack or brute forcing permissions to elevate to administrative/higher access rights.

6.8 Compare and contrast remote exploit and a local exploit

Local exploit is when an attacker has access to the vulnerable system

Remote exploit takes place over a network with no prior access.

Exploit kits can be utilised to facilitate this.